Human arm endpoint-impedance in a rhythmic human-robot interaction exhibit cyclic variations

Vincent Fortineau*1,2,3

Isabelle A. Siegler*2,3

Maria Makarov1

Pedro Rodriguez-Ayerbe1

1 - Laboratoire des signaux et systèmes (L2S), Université Paris-Saclay, CNRS, CentraleSupélec, 91190, Gif-sur-Yvette, France

2 - CIAMS, Université Paris-Saclay, 91405 Orsay Cedex, France

3 - CIAMS, Université d'Orléans, 45067, Orléans, France

Corresponding author :

Vincent Fortineau: vfortineau@free.fr

ORCiD : 0000-0003-4341-6615

Isabelle A. Siegler: isabelle.siegler@universite-paris-saclay.fr

ORCiD : 0000-0002-5140-7682

P. Rodriguez-Ayerbe and M. Makarov supervised the work on the robotic control strategies, the identification methodology for impedance estimation, the trajectory estimation and made corrections for this article.

I. A. Siegler supervised the work on the experimental protocol, the variance analysis and strongly contributed to the redaction of this article.

V. Fortineau designed the experimental test bed and the impedance methodology, conducted the experiments and data analysis, and mainly contributed to the redaction of this article.

Running head title : Human arm impedance in a rhythmic human-robot interaction

# Abstract

## Estimating the human endpoint-impedance interacting with a physical environment provides insights for goal-directed human movements during physical interactions. This work examined the endpoint-impedance of the upper limb during a hybrid ball-bouncing task with simulated haptic feedback while participants manipulated an admittance-controlled robot. Two experiments implemented a force-perturbation method to estimate the endpoint parameters of 31 participants. Experimental conditions of the ball-bouncing task were simulated in a digital environment. One experiment studied the influence of the target height, while the other studied the impedance at three cyclic phases of the rhythmic movement induced by the task. The participants’ performances were analyzed, clustered, and compared with estimated impedance parameters in order to establish a potential influence of endpoint-impedance on performance in the ball-bouncing task. Results showed that endpoint-impedance parameters ranged from 45 to 445 N/m, 2.2 to 17.5 Ns/m, and 227 to 893 g for the stiffness, damping, and mass, respectively. Results did not support such a critical role of endpoint-impedance in performance. Nevertheless, the three endpoint-impedance parameters described significant variations throughout the arm cycle. The stiffness is linked to a quasi-linear increase, with a maximum value reached before ball impacts. The observed damping and mass cyclic variations seemed to be caused by geometric and kinematic variations. Although this study reveals rapid and within-cycles variations of endpoint-impedance parameters, no direct relationship between endpoint-impedance values and performance levels in ball-bouncing could be found.

## Keywords : Endpoint-impedance, Human arm stiffness, Physical human-robot interactions, Rhythmic movements, Ball bouncing

## News & Noteworthy

We examined the arm's endpoint-impedance of 31 participants in a dynamic rhythmic task during physical human-robot interactions without constraint about the participants' joints. We obtained good identifications indicating significant rhythmic variations of the endpoint-impedance parameters. In particular, we observed an increase of stiffness throughout the cycle, supposedly in preparation for ball impact. There was no significant difference in impedance parameters between different performance level groups, suggesting a minor influence of endpoint-impedance on task performance.

# Introduction

Humans can tailor their limbs’ dynamic properties to achieve stable behavior with a wide range of environments while simultaneously ensuring dexterous performances (1–3). Measuring the properties of the humans’ interactions during various tasks is relevant to understand the emergence of perturbed goal-directed movements. Reproducing these behaviors applies to the design of bio-inspired robot control and can also help gain insight into human motor control.

Among the many dynamic properties of limb, researchers have studied the mechanical impedance for decades under the control prism (4–6). Impedance relates kinematics to forces, allowing the modeling of human movements during physical interactions. Despite their limitations (7), elementary models, such as mass-spring-damper models (KBM or KBI) or Hill-type muscle models, have been shown to approach human behavior (3) and performances (8). These models do not intend to explain the complete human motor control but rather reproduce some of its aspects. More comprehensive models are required to analyze the neural processes underlying the control of dynamic limbs’ properties (9, 10). To account for passive muscle viscoelastic properties and active behaviors induced by reflexes and motion, Latash and Zatsiorsky (11) explained that a single joint would at least need 13 parameters, making it very sophisticated and almost impossible to study multiple joints impedance during movements. Less elaborated models at the endpoint level have been widely studied (1, 3, 12, 13), notably with regard to impedance during movements of multiple joints (1, 13). Decoupling simple models from biological considerations to study movements at the endpoint level with a behavioral approach even allowed researchers to analyze performances and expertise (13), learning (1, 8), and exploration (14).

Studying the mechanical impedance is strenuous, however, since it cannot be directly measured. Estimating impedance requires perturbing the limbs and comparing the observed movement to an unperturbed trajectory. Two configurations can be easily distinguished: static or isometric tasks (12, 15), where the unperturbed pose is well known, and dynamic tasks, where the estimation of the unperturbed trajectory is much more complex (16, 17). When studying dynamic tasks, free movements without physical interaction with the environment should be distinguished from hybrid movements for which interactions with the environment are intermittent, potentially requiring time-varying mechanical impedance.

The study of impedance variations during movements and notably cyclic movements with contact phases and hybrid transitions is especially relevant for locomotion. The literature has thoroughly studied time-varying stiffness about the ankle joint, notably with walking (18) or running (19). Guarin (9) also studied the reflex and intrinsic viscoelastic properties contributions of the ankle time-varying stiffness. To provide insights into the flexibility of human walking, Huang and Wang (20) designed a Central Pattern Generator (CPG)-based human locomotion model, where the CPG model controlled not only joint torque but also joint stiffness (stiffness constant and equilibrium point). They improved the disturbance rejection from the ground ruggedness compared to previous models. The time-varying viscoelastic properties of the upper limbs have also been investigated in free cyclic movements (21) and rhythmic movements with various loads (22). Lacquaniti et al. (23) and Tsuji and Tanaka (3) studied the endpoint-impedance of a ball-catching hybrid task (with both free movements and contact phases) with predictable physical interactions.

The present study characterizes the upper-limb mechanical impedance in a hybrid rhythmic task, such as a ball-bouncing task. The ball-bouncing task has already been studied for its apparent simplicity in order to address several motor control issues  (24–28). Avrin et al. (29) implemented a CPG-based control structure to reproduce human-like kinematic performances in the ball bouncing. However, in the experimental data used to validate this model (26, 27), the dynamics of the physical interactions between the ball, the paddle, and the human were not simulated. A new experimental setup designed in previous experiment (30) overcame this shortcoming using an admittance-controlled robot  that participants manipulated in order to vertically move a virtual paddle and bounce a virtual ball in a digital environment. The haptic feedback was also simulated during ball-paddle impacts;  it is relevant for performances, as it reduces the noise in the state estimation of the ball (24). Later studies (31, 32) validated the methodology for endpoint-impedance estimation during cyclic movements. Two experiments described in the present study evaluated endpoint-impedance in such an ecological multijoint rhythmic movement with low constraints generated by the robot. The first goal of the experiment was to assess whether endpoint-impedance parameters vary and follow a cyclic behavior. The second goal was to study both the potential influence of the ball height to be reached on endpoint-impedance and a possible relation between endpoint-impedance parameters and performance level, as suggested by previous research on this topic (13).

# Materials and Methods

## Participants

Thirty-one healthy participants with no known motor disorders took part in the experiment. Among the participants, 25% were women, 10% were left-handed, 40% had already manipulated a similar robotic system, and 30% had never interacted physically with a robot. On average (SD standard deviation), participants were 30.0 years old (SD 8.6), declared a height of 176.2 cm (SD 9.3), and weighed 71.3 kg (SD 12.7). Before the experiment, participants were required to read and sign a written informed consent form approved by the Ethics Committee for Research (CER) of the Université Paris-Saclay (file number 217, September 2020). The Ethics Committee also approved the experimental procedure.

## Apparatus

Figure 1 depicts the experimental setup used for the experiment. Participants were standing in front of a table supporting a KUKA youBot 5 DOF robot. The robot had its first and last joints position-controlled to a fixed orientation to evolve only in the participants’ sagittal plane. The other three joints were admittance-controlled at 1 kHz, as detailed in Fortineau et al. (30), so that participants could move the robot along a vertical line without feeling the robot’s weight, joints’ friction and with reduced apparent inertia (33). Forces of interaction were measured using a 6-axis force/torque sensor ATI Mini 45, placed between the robot endpoint and a handle that participants maneuvered. The data from the sensor was sampled at 1 kHz. For the control, the endpoint position of the robot was obtained using joint data. However, to limit bias from joint flexibility, a motion capture system V120:Trio by Optitrack measured handle position with a submillimetric accuracy at 120 Hz. This position measurement was used for data analysis.

Figure 1 THERE

Figure 1: Experimental setup.

## Ball-bouncing task

Participants had to operate the robot handle in order to move a paddle in a digital environment where the ball-bouncing task was simulated. The ballistic equation determined the ball kinematics during free movement, and the ball post-impact velocity used a restitution coefficient α=0.6 to account for energy loss, as described in Fortineau et al. (30).

The goal of the task was to match the ball apex position with a target height h*, kept constant during each trial and materialized by a horizontal line. A score was displayed slightly above the target line to help participants reduce the bouncing error ϵr between the ball center at the apex and h*. Participants were instructed to try to maintain a constant ball impact position zk during trials. During both experiences, the robot introduced perturbations timed from the ball impacts on the simulated paddle at three possible cyclic phases: (φ1) after the ball impact, (φ2) during the decreasing phase of the paddle, and (φ3) before the subsequent ball impact near the lowest position. These phases are shown in Figure 2. Perturbations were randomly spaced between 2 and 5 s.

Figure 2 THERE

Figure 2: Phases perturbed along a time normalized cyclic position trajectory (the colored area represents the standard deviation).

### Haptic feedback & Perturbations

The robot admittance control loop was interrupted for 30 ms, and a force fi along the vertical axis was introduced using only the torque control loop, with a constant setpoint to simulate ball impacts. The equation (1) adapted from Kawazoe (34) determined the magnitude of the force, with b and p indicating either the ball or the paddle, v the velocity right before the impact k, m the mass, and  K=650 an estimated coupled stiffness between the arm and the ball.

|  | fik=-vbk-vpk1+αmb+Kπ1+mbmp | (1) |
| --- | --- | --- |

Perturbations were introduced through the same pathway in stochastic directions. Figure 3 shows an example of two resulting endpoint force perturbations. The peak value is reached approximately 30 ms after the onset of the torque control, and the next front lasts less than 30 ms afterward. The total duration of a perturbation is therefore below 60 ms.

Figure 3 THERE

Figure 3: Two examples of force perturbations resulting from the introduction of torque controlled perturbations.

## Mechanical impedance modeling

Equation (2) shows the well-known KBM model used in this study.

|  | δf=Kδx+Bδx+Mδx | (2) |
| --- | --- | --- |

This model relates a differential force δf to a differential position δx when a perturbation force δf is introduced along the virtual trajectories f0 and x0. The differentials are the differences between virtual and perturbed trajectories (measured). The model can be seen as a perturbation rejection system with the linear parameters K, B, and M: the stiffness, damping, and mass, respectively. Virtual trajectories (35) refer to completely unperturbed behaviors and are approximated using the nominal trajectories in the absence of the perturbations described in the previous section. The model uses its decoupled form, which was already applied by Erden and Billard (13, 36). This work only study the impedance along the movement.

The virtual paddle position was approximated using spline interpolation with a window of 350 ms, chosen to minimize the quadratic error with an undisturbed trajectory and maximize the duration to account for the arm’s time response. The virtual force was estimated using a sum of sines optimization with a masked window of 100 ms after the perturbation (32). Figure 4 shows an example for both trajectories.

Figure 4 THERE

Figure 4: Position and force trajectories (blue), with the estimated virtual trajectories (red).

The impedance parameters were then estimated using an ARX identification on a discretized impedance model on an identification window of 150 ms. This window was minimal while still allowing proper identification (32). Automatic responses or voluntary actions can occur with 100 to 180 ms delays (37, 38) and cannot be modeled with a linear impedance model. They were, however, considered not dominant even after 200 ms (13, 36).

## Experimental protocol

Two experiments (Experiment P, Experiment H) occurred during two sessions on the same day (for all but three participants) separated by at least 45 min. Each session lasted 30 min on average. The first session (Session 1) consisted of a familiarization phase and then two trials of 5 min each with a brief pause in between. Participants started the first trial when confident enough with the ball-bouncing task. The second session (Session 2) consisted of three other trials of 5 min each, also separated by small pauses to limit fatigue.

Experiment P consisted of three trials: either the two trials of Session 1 and the last trial of Session 2, or the three trials of Session 2. The target height h2* was fixed at 1.75m, and the three cyclic phases φ were randomly perturbated in all trials.

Experiment H consisted of two trials (the two trials of Session 1 or the two first trials of Session 2). The two trials randomly attributed a target height of either 1.5 m (h1*) or 2 m (h3*). In this experiment, all perturbations occurred at phase φ1.

Participants were randomly selected to start with Experiment H (17 participants) or Experiment P (14 participants), which parted the participants into two experiment-order subgroups. After each experiment, participants were asked to report their perception of haptic feedback and perturbations.

## Data analysis

With 31 participants, 155 trials led to more than 14h of recorded data, not including the familiarization phase. All acquired data was interpolated on a time vector sampled at 1 kHz. The position and force were filtered using Butterworth second-order digital filters with cutoff frequencies of 25 Hz and 50 Hz, respectively. Cycles were defined using ball impacts.

We distinguished three performance levels in both experiments’ data, defined as novice e1, intermediate e2, and advanced e3. The individual performance was first evaluated using the interquartile range of the bouncing errors ϵr on each participant’s data, and the median of the quadratic bouncing errors ϵr2 as first measures of repeatability. These groups were generated using a k-means clustering algorithm on the interquartile range of ϵr since repeatability was proven to qualify performances (39).

For the rest of the analyses on performance, two dependent variables were used to characterize the task, using the same method as previous studies on ball bouncing. The first variable defined the precision with the average bouncing error ϵr. The second variable defined the repeatability using the standard deviation of the bouncing errors σ(ϵr).

On average, 32.4 (SD 6.1) impedance parameter identifications for each experiment, configuration (h* and φ), and participant were used to estimate the endpoint-impedance parameters. Medians were computed for every parameter since they proved to be more accurate in simulations than average values after taking out outliers. Each impedance parameter was populated with 93 estimations (31 participants × 3 phases) for Experiment P and with 62 estimations for Experiment H (31 participants × 2 target heights).

The adequation of the impedance parameters of each identification was evaluated using the coefficient of determination R2 of the reconstruction of the perturbed position trajectory with the identified parameters and the differential force trajectory.

We used JASP (version 0.16) to perform the analyses of variance (ANOVA) considering a threshold of 5%. We also conducted post-hoc tests using the Bonferroni correction. Only significant results are detailed in the result section. We used Matlab (release 2020b) to process data.

# Results

## Effect of perturbations on participants’ performance

Perturbations were programmed to be as seamless as possible while providing sufficient deviations for impedance estimations (32). Even if participants were warned of both haptic feedback and perturbations, only 55% declared having felt perturbations during at least one of the trials.

All the bouncing errors taking place the cycle before a perturbation c-1 and three cycles after (c1, c2, c3) were averaged for each participant and compared using a mixed-design ANOVA (4 cycles × 2 experiments) to assess the effect of perturbations on performance. Only a significant effect of cycle order on ϵr was observed F(3, 180) = 6.8, p<0.001, η2=0.026. A post-hoc analysis revealed that the bouncing error at c1 was significantly larger than other cycles’ errors, with a mean difference of 2.0 cm (Figure 5).

Figure 5 THERE

Figure 5: Mean bouncing error ϵr  as a function of cycle order, with standard errors used as confidence intervals.

A second mixed-design ANOVA conducted on σϵr with the same factors had to be corrected (40) since the sphericity test was not verified (41). No significant effect was observed for the main effect factor, the random effect, or the interaction. Therefore the repeatability of the bouncing error was similar for all cycles, with an average standard deviation of 25.1 cm (SD 5.0).

## Performance-based clustering of participants

Among all participants, 40% were already familiar with the experimental setup, while 50 % had never manipulated an articulated robot. K-means analysis showed that one participant had to be discarded from the computation of group performances because their score was too low, thus putting the participant in their own group. The clustering was populated with 5 advanced, 13 intermediate, and 13 novice users. Interestingly, a similar clustering was obtained using the quadratic errors. Only 3 participants were classified differently.

To validate the essence of the clustering, the differences between performance-level groups ei were studied using two one-way ANOVAs, on the precision using the mean bouncing error ϵr and on the repeatability using the standard deviation σ(ϵr). The ANOVA on ϵr unveiled no significant effect of the level-group factor F(2,28)=3.0, p=0.064, η2=0.178, while the ANOVA on  σ(ϵr) revealed a strong and significant effect of the performance-level factor F2,28=33.0,  p<0.001,  η2=0.702.

The paddle acceleration at impact (28, 42) offers another way of looking at individual performances. However, the accelerations estimated with two points numerical derivatives were not significant between the performance-level groups. Participants hit the ball with an average post-impact acceleration of 0.29 ms-2  (SD 1.69).

## Experiment H

### Performances

For target heights h1*= 1.5m and h3*=2m, the mean ball-bouncing magnitudes (difference between apex and impact position) were 1.176 m (SD 0.019) and 1.670 m (SD 0.019), respectively. These values show that participants hit the ball at similar vertical positions in both conditions and did not tend to hit it at a higher position when the target was higher.

The effect of target height hi* and performance-level groups ei on ϵr was analyzed using a mixed-design ANOVA (between: 2 × within: 3), revealing a significant main effect of factor hi*, F1,28=7.0, p=0.014, η2=0.043. Neither the effect of performance level ei nor the interaction was significant. Overall, the mean errors for h1* and h3* were 6.8 cm (6.3 SD) and 3.2 cm (7.1 SD), respectively.

On the whole group, the mean σϵr increased from 21.7 cm (SD 4.2) to 27.2 cm (SD 6.1) for h1* and h3*, respectively. A mixed ANOVA (between: 2  hi* × within: 3 ei) revealed a significant main effect of hi*, F1,28=44.8, p<0.001, η2=0.179, a significant effect of ei F1,28=19.6, p<0.001, η2=0.404, but no significant interaction. The post-hoc analysis revealed that the level of performance was significantly different for each pair. Figure 6 details the variations of the mean σϵr according to both factors.

Figure 6 THERE

Figure 6: Mean repeatability σ(ϵr) according to performance group-levels and target heights, with standard errors used as confidence intervals. Groups that are not significantly different are marked with the same letter for each target height.

### Endpoint-impedance parameters

The same factors hi* and ei were analyzed for each endpoint-impedance parameter. ANOVAs did not yield any significant results for the stiffness nor for the damping. The average estimated stiffness was 116 N/m (SD 60), with a minimum value of 45 N/m and a maximum of 401 N/m. The average estimated damping was 9.9 Ns/m (SD 3.1), with a minimum value of 2.2 Ns/m and a maximum of 16.8 Ns/m.

The mixed ANOVA about the estimated mass revealed a significant effect of the main factor hi*, F1,28=4.6, p=0.040, η2=0.014. Neither the effect of performances ei nor the interaction was significant. The average mass estimated for h1* was 459 g (SD 126) and 434 g (SD 94) for h2* with a minimum value of 272 g and a maximum of 893 g.

## Experiment P

### Performances

The one-way ANOVA for each variable used to characterize performances was conducted on the regrouped data of the three trials, studying the effect of performance clusters.

The ANOVA about the average bouncing error ϵr revealed no significant effect of the performance clusters ei. The mean error for h2* (Experience P) was 5.4 cm (SD 5.1). The ANOVA about the standard deviation of the error σ(ϵr) revealed a significant effect on the performance clusters ei, F2,28=24.0, p<0.001, η2=0.631. The average standard deviation was 27.8 cm (SD 3.6), 22.7 cm (SD 2.1), and 18.2 cm (SD 2.0), respectively for the clusters e1, e2, and e3. The post-hoc analysis showed that each pair had significant differences.

Figure 7 THERE

Figure 7: Mean precision $ϵr and repeatability σ(ϵr) for the data of experiment P (h2*) as a function of performance clusters, with standard errors used as confidence intervals. Groups that are not significantly different are marked with the same letter for each ANOVA.

### Endpoint-impedance parameters (cyclic phase factor)

The factors φi and ei were analyzed for each endpoint-impedance parameter estimated using mixed-design ANOVAs (between: 3 × within: 3).

The mixed ANOVA about the endpoint stiffness revealed a significant effect of φi, F2,56=8.3, p<0.001, η2=0.134. Neither the effect of performances ei nor the interaction was significant. The post-hoc analysis indicated that the variations were significant for the couple (φ1,φ3). The couple (φ1,φ2), indicated a significant variation with Holm correction but not with Bonferroni correction. The stiffness progressively increased from φ1 125 N/m (SD 78) to φ3 191 N/m (SD 64), as shown in Figure 8-i. The stiffness values were bounded between 53 N/m and 445 N/m.

The mixed ANOVA about the endpoint damping revealed a significant effect of φi, F2,56=8.6, p<0.001, η2=0.138, and of performances ei F2,56=6.4, p=0.005, η2=0.123. No significant interaction was observed. The post-hoc analysis indicated that the variations were significant for the couples (φ1,φ2) and (φ2,φ3)  with an average difference of 1.9 Ns/m and 2.2 Ns/m, respectively. Moreover, the clusters of performances were significantly different for the couples (e1,e2) and (e2,e3). All damping values were bounded between 3.5 Ns/m and 17.5 Ns/m. The cyclic behavior of the damping is shown in Figure 8-ii.

The mixed ANOVA about the apparent mass used Huynh-Feldt’s (1976) correction because the sphericity assumption was violated (p<0.05). It revealed a significant effect of the main factor φi, F1.5,41.3=16.2,  p<0.001,  η2=0.063. Neither the effect of performances ei nor the interaction was significant. The post-hoc analysis indicated that the variations were significant for the couples (φ1,φ2), with an average difference of 71 g, and (φ2,φ3), with an average difference of 67 g. All mass values were bounded between 227 g and 815 g. The cyclic behavior of the mass is shown in Figure 8-iii.

Figure 8 THERE

Figure 8: Mean estimated impedance parameters (K, B, and M), according to the phase perturbed (and expertise when significant differences were observed), with standard errors used as confidence intervals. Groups that are not significantly different are marked with the same letter for each ANOVA.

The mixed ANOVA about the coefficients of determination used Huynh-Feld’s correction since the sphericity assumption was violated (p<0.05). It revealed a significant effect of the main factor φi, F1.6,45.9=31.1,  p<0.001,  η2=0.302. Neither the effect of performances ei nor the interaction was significant. The post-hoc analysis indicated that the variations were significant for the couples (φ1,φ2), with an average difference of 3.3%, and (φ1,φ3), with an average difference of 2.6%. Phase φ1 had the lowest average score with 93.5% (+/-2.7%). The minimum users’ coefficient of determination (88.9%) was obtained at phase 1 and the maximum (98.6%) at phase 3.

The factors φi and ei were analyzed for each endpoint-impedance parameter notch interval (median 95% confidence interval) using mixed-design ANOVA (between: 3 × within: 3).

The mixed ANOVA about the endpoint stiffness variability used Huynh-Feld’s correction since the sphericity assumption was violated (p<0.05), revealing a significant effect of φi, F1.4,39.0=4.8, p=0.024, η2=0.094. Neither the effect of performances ei nor the interaction was significant. The post-hoc analysis indicated that the variations were significant for the couple (φ1,φ2), with a decrease from 89 N/m (SD 58) to 52 N/m (SD 20). The phase φ3 had an average notch of 78 N/m (SD 32).

The mixed ANOVA about the endpoint damping variability revealed a significant effect of φi, F2,56=3.5, p=0.037, η2=0.041, and the interaction F4,56=2.7, p=0.037, η2=0.065. However, the effect of performances ei was not significant. The post-hoc analysis indicated that the variations were significant for the couple (φ1,φ2), with a decrease of 0.5 Ns/m (SE 0.2). The damping notches ranged from 1.1 Ns/m to 4.8 Ns/m.

The mixed ANOVA about the endpoint mass variability used Huynh-Feld’s correction since the sphericity assumption was violated (p<0.05). It revealed a significant effect of φi, F1.4,39.0=8.0, p=0.004, η2=0.114. Neither the effect of performances ei nor the interaction was significant. The post-hoc analysis indicated that the variations were significant with phase φ2 and both other phases, with a lower notch value of 32 g (SD 16), compared to 56 g (SD 33) and 55 g (SD 25) for φ1 and φ3, respectively.

# Discussion

This work focuses on analyzing the endpoint-impedance of the upper limb during a ball bouncing task with haptic feedback. The results of the experiments offers insights for the two main questions raised, 1) looking for relation between performance and the endpoint-impedance behavior and, 2) analyzing the endpoint-impedance variations during a cyclic task. Results are compared with the literature, and the perturbations’ influence is discussed to comfort the methodology.

## Impedance estimation validation during a ball bouncing task

As presented in the section “Effect of perturbations on participants’ performance”, only the cycles after a perturbation unveiled a significant mean error ϵr increase of about 2.0 cm on average. This increase can be considered negligible with respect to the mean bouncing magnitude of 1.42 m and the standard deviation of the bouncing error σϵr, around 25 cm for all participants. Moreover, the ANOVA on σϵr did not underline significant differences between the cycles, showing that their repeatability did not deteriorate. Therefore, the methodology used to introduce these perturbations is close to the main objective of total transparency or seamlessness, which could allow online impedance estimation without disturbing the studied task.

The endpoint-impedance parameters estimated ranged from 45 to 445 N/m, 2.2 to 17.5 Ns/m, and 227 to 893 g for the endpoint stiffness, damping, and mass, respectively. No constraints were imposed on the participants, thus inducing the involvement of at least three joints (wrist, elbow, shoulder) to whom the hip joint might be added. A projection of 52 (13 × 4) parameters (without considering coupling effects between joints) would have been required for joint impedance analysis. With only three parameters, 94.6 % (SD 2.9) of the observed behavior could be reproduced on average for the case of slight deviations from trajectory imposed by a rhythmic task, pointing out the adequacy of the linear impedance model used.

Moreover, the endpoint parameters obtained from the linear model are comparable to those found in the literature. For point-to-point movements, Burdet et al. (1) found higher stiffness parameters ranging from 150 to 700 N/m. In the preparation of ball impacts, Tsuji and Tanaka (3) found stiffness from 44 to 189 N/m, and in a collaborative welding task, Erden and Billard (13) found similar stiffness values from 49 to 339 N/m in the direction of the movement. This study’s apparent endpoint damping and mass are slightly lower than Tsuji and Tanaka’s (3) and Erden and Billard’s (13) observations. Lower damping values might be explained by the kinematic requirements of the ball-bouncing task and lower masses by the geometric configuration imposed by the experimental setup.

## Are there cyclic variations of the endpoint-impedance parameters?

In the preparation of a predictable contact with an environment, joint stiffness in the case of the ankle for locomotion (18) or elbow and wrist in a ball catching task (23) increased prior to the contact. Tsuji and Tanaka (3) also found an increase in stiffness at the endpoint level right before motion compared to a stable posture. They deduced that the impedance regulation occurred before starting motions for a task. However, there are some evidences that could imply neurophysiological differences in the emergence of rhythmic movement in opposition to discrete or point-to-point movements (43, 44), even if unified theories have been proposed (45, 46). Consequently, impedance variations results during discrete tasks might not transfer to rhythmic movements. The results of the endpoint-impedance estimation described in this study also indicate significant variations of the impedance parameters at three key point phases of the ball bouncing cycle.

The lowest stiffness values are obtained at phase φ1, which takes place 0.3 s after the impact and progresses to reach the highest values right before the impact at phase φ3, 0.3 s before the impact. As detailed in the result section, this increase is almost linear throughout the three phases, with a significant growth of 53% between φ1 and φ3, implying smaller displacements when subject to equivalent force.

Counter-intuitively, the endpoint damping reached its lowest values at φ3 (no significant difference with φ1) and highest value in the middle of the decreasing phase at φ2, with significant differences from the other two phases. One might have expected a behavior correlated with the endpoint stiffness to improve perturbation rejection; in fact, the damping factor of a second-order model is proportional to the damping B of the impedance model, and the settling time of such a model is inversely proportional to the damping factor. If we consider the average parameters of all participants for each phase, the settling time in response to a force pulse of both φ1 and φ3 is 0.43 s, while it is about 0.34 s for φ2. Therefore, at phase φ2, the arm returns faster to its equilibrium position. Endpoint damping variations seem uncorrelated with the expected requirements imposed by the hybrid task since an increase in the time response would decrease the perturbation rejection performance.

As previously stated, the identifications are conducted on 150 ms; the assumption of constant parameters was also made during that time frame. Even with this assumption, significant variations are observed within 0.6 s. Because of the assumption of constant parameters, parameters estimated at φ1 would be shifted away from the perturbation (since they are averaged on 150 ms) while shifted closer to the perturbation for φ3, making both phases positioned in a positive acceleration cyclic phase, as opposed to φ2. Therefore, damping variation could be related to velocity or the sign of acceleration.

The increase in response time to perturbation rejection for the phases close to a predictable perturbation might notify involuntary impedance variation. Indeed, it has been widely reported that joint impedance is related to the joint configuration and velocity (47). To reject this latter hypothesis, an experience involving similar kinematics but varying interaction force could be tested by changing the ball mass.

## Can endpoint-impedance parameters explain performance?

During a point-to-point task, Burdet et al. (1) reported a selective increase of stiffness in the direction of instability generated by a force field but a low impedance in the direction of the movement. As explained by the authors, since co-contractions have a high metabolic cost and full central nervous system (CNS) control is computationally costly, the CNS is facing an optimization problem to enhance robustness in the direction of perturbation while minimizing metabolic cost. After some trials, the adaptation of the stiffness in the absence of the destabilizing force underlined the learning of the optimal endpoint-impedance. Therefore the CNS can adapt not only the endpoint stiffness magnitude but also the endpoint stiffness geometry (shape and orientation) in a predictive way that is independent of the force required to compensate for the newly imposed dynamics.

In two other experiments, Erden and Billard (13, 36) demonstrated differences in endpoint-impedance. A first experiment compared handedness skills, and a second compared performances between expert and novice participants. They found significant expertise effects on both endpoint stiffness and damping. Novice users tended to have lower average impedance parameters. Similar results were obtained for handedness.

These experiments emphasize the importance of adapting the endpoint-impedance parameters during movements and the relationship between these parameters and expertise or skill. Therefore, it would be expected to find significant differences while looking at endpoint parameters according to performance levels.

However, despite significant performance differences between the three levels chosen, most endpoint-impedance parameters were not significantly different. There might be two main explanations. A first explanation could involve the strategies used by participants to improve their performances. Morice et al. (42) and de Rugy et al. (25) reported that the ball bouncing task might be achieved outside of a passively stable regime thanks to an active control relying on visual information about the ball. Compared to kinematic and timing, our findings could underline a minor role of viscoelastic endpoint properties in this task. If the impedance is not critical to the task, its variability might increase. The participant’s endpoint acceleration at the ball’s impacts was on the verge of active and passive control and did not allow conclusions to be drawn. It is also important to stress that no significant impact acceleration variation was found between the different performance levels, suggesting that all participants used both passive and active control. To dismiss the hypothesis that impedance variations are not critical to performances in the ball bouncing task, an experiment with variations of dynamic considerations could be proposed, using the change of the gravity constant or the ball mass.

A second explanation for the endpoint-impedance observations could involve the proposed protocol. Even if some participants had prior experience with the experimental setup, the performance clusters were not defined according to expertise like Erden and Billard (13); it was instead done retrospectively. Therefore, the clusters chosen, even if significantly different, might not provide important enough differences. The variations caused by performance need to be more critical than individual variations to be statistically significant. To be able to observe slighter variations and reduce noise in the estimated parameters, the position trajectories’ precision could be enhanced, or more participants could be included.

## Conclusion

In realistic case studies, joint impedance might be complicated to estimate when human movements are free and the environment is partially unknown. The use of endpoint-impedance provides a good approximation for small deviations from a nominal trajectory, allowing the monitoring of a global behavior. Our study in particular did not discriminate reflexes from intrinsic properties.

The experiments described above revealed significant rapid impedance variations (<0.6 s) during the ball bouncing task, supporting the idea of cyclic variations of the endpoint-impedance during rhythmic tasks. Estimation techniques not relying on the assumption of constant parameters like the ones described by Piovesan et al. (48) or Guarin and Kearney (9) could provide more specific and descriptive cyclic impedance data in order to both draw more delicate variations and consolidate the findings of this work.

# Acknowledgments

We would like to thank Baptiste Boyer, Thomas Chevet, Dario Penco, Jeremy Pinguet and Martin Soyer who helped for the tuning of the experiment.

# Disclosure

No conflicts of interest, financial or otherwise, are declared by the authors.

# References

1. 	Burdet E, Osu R, Franklin DW, Milner TE, Kawato M. The central nervous system stabilizes unstable dynamics by learning optimal impedance. Nature 414: 446–449, 2001. doi: 10.1038/35106566.

2. 	Milner TE, Franklin DW. Impedance control and internal model use during the initial stage of adaptation to novel dynamics in humans. The Journal of Physiology 567: 651–664, 2005. doi: 10.1113/jphysiol.2005.090449.

3. 	Tsuji T, Tanaka Y. Bio-mimetic impedance control of robotic manipulator for dynamic contact tasks. Robotics and Autonomous Systems 56: 306–316, 2008. doi: 10.1016/j.robot.2007.09.001.

4. 	Mussa-Ivaldi FA, Hogan N, Bizzi E. Neural, mechanical, and geometric factors subserving arm posture in humans. J Neurosci 5: 2732–2743, 1985.

5. 	Börner H, Endo S, Hirche S. Estimation of Involuntary Components of Human Arm Impedance in Multi-Joint Movements via Feedback Jerk Isolation. Frontiers in Neuroscience 14: 459, 2020. doi: 10.3389/fnins.2020.00459.

6. 	van de Ruit M, Mugge W, Cavallo G, Lataire J, Ludvig D, Schouten AC. Quantitative comparison of time-varying system identification methods to describe human joint impedance. Annual Reviews in Control 52: 91–107, 2021. doi: 10.1016/j.arcontrol.2021.10.010.

7. 	Kistemaker DA, Rozendaal LA. In Vivo Dynamics of the Musculoskeletal System Cannot Be Adequately Described Using a Stiffness-Damping-Inertia Model. PLOS ONE 6: e19568, 2011. doi: 10.1371/journal.pone.0019568.

8. 	Tee KP, Burdet E, Chew CM, Milner TE. A model of force and impedance in human arm movements. Biol Cybern 90: 368–375, 2004. doi: 10.1007/s00422-004-0484-4.

9. 	Guarin DL, Kearney RE. Unbiased Estimation of Human Joint Intrinsic Mechanical Properties During Movement. IEEE Transactions on Neural Systems and Rehabilitation Engineering 26: 1975–1984, 2018. doi: 10.1109/TNSRE.2018.2870330.

10. 	Sobhani Tehrani E, Jalaleddini K, Kearney RE. Ankle Joint Intrinsic Dynamics is More Complex than a Mass-Spring-Damper Model. IEEE Transactions on Neural Systems and Rehabilitation Engineering 25: 1568–1580, 2017. doi: 10.1109/TNSRE.2017.2679722.

11. 	Latash ML, Zatsiorsky VM. Joint stiffness: Myth or reality? Human Movement Science 12: 653–692, 1993. doi: 10.1016/0167-9457(93)90010-M.

12. 	Dolan JM, Friedman MB, Nagurka ML. Dynamic and loaded impedance components in the maintenance of human arm posture. IEEE Transactions on Systems, Man, and Cybernetics 23: 698–709, 1993. doi: 10.1109/21.256543.

13. 	Erden MS, Billard A. Hand Impedance Measurements During Interactive Manual Welding With a Robot. IEEE Transactions on Robotics 31: 168–179, 2015. doi: 10.1109/TRO.2014.2385212.

14. 	Li Y, Jarrassé N, Burdet E. Versatile Interaction Control and Haptic Identification in Humans and Robots. In: Geometric and Numerical Foundations of Movements, edited by Laumond J-P, Mansard N, Lasserre J-B. Springer International Publishing, p. 187–206.

15. 	Fang C, Ajoudani A, Bicchi A, Tsagarakis NG. Online Model Based Estimation of Complete Joint Stiffness of Human Arm. IEEE Robotics and Automation Letters 3: 84–91, 2018. doi: 10.1109/LRA.2017.2731524.

16. 	Burdet E, Osu R, Franklin DW, Yoshioka T, Milner TE, Kawato M. A method for measuring endpoint stiffness during multi-joint arm movements. Journal of Biomechanics 33: 1705–1709, 2000. doi: 10.1016/S0021-9290(00)00142-1.

17. 	Gomi H, Kawato M. Human arm stiffness and equilibrium-point trajectory during multi-joint movement. Biol Cybern 76: 163–171, 1997. doi: 10.1007/s004220050329.

18. 	Lee H, Rouse EJ, Krebs HI. Summary of Human Ankle Mechanical Impedance During Walking. IEEE Journal of Translational Engineering in Health and Medicine 4: 1–7, 2016. doi: 10.1109/JTEHM.2016.2601613.

19. 	Shorter AL, Rouse EJ. Ankle Mechanical Impedance During the Stance Phase of Running. IEEE Transactions on Biomedical Engineering 67: 1595–1603, 2020. doi: 10.1109/TBME.2019.2940927.

20. 	Huang Y, Wang Q. Disturbance rejection of Central Pattern Generator based torque–stiffness-controlled dynamic walking. Neurocomputing 170: 141–151, 2015. doi: 10.1016/j.neucom.2015.04.096.

21. 	Bennett DJ, Hollerbach JM, Xu Y, Hunter IW. Time-varying stiffness of human elbow joint during cyclic voluntary movement. Exp Brain Res 88: 433–442, 1992. doi: 10.1007/BF02259118.

22. 	Li K, Liu X, Zhang J, Zhang M, Hou Z. Continuous motion and time-varying stiffness estimation of the human elbow joint based on sEMG. J Mech Med Biol 19: 1950040, 2019. doi: 10.1142/S0219519419500404.

23. 	Lacquaniti F, Carrozzo M, Borghese NA. Time-varying mechanical behavior of multijointed arm in man. Journal of Neurophysiology 69: 1443–1464, 1993. doi: 10.1152/jn.1993.69.5.1443.

24. 	Ankarali MM, Tutkun Şen H, De A, Okamura AM, Cowan NJ. Haptic feedback enhances rhythmic motor control by reducing variability, not improving convergence rate. Journal of Neurophysiology 111: 1286–1299, 2014. doi: 10.1152/jn.00140.2013.

25. 	de Rugy A, Wei K, Müller H, Sternad D. Actively tracking ‘passive’ stability in a ball bouncing task. Brain Research 982: 64–78, 2003. doi: 10.1016/S0006-8993(03)02976-7.

26. 	Siegler IA, Bardy BG, Warren WH. Passive vs. active control of rhythmic ball bouncing: the role of visual information. J Exp Psychol Hum Percept Perform 36: 729–750, 2010. doi: 10.1037/a0016462.

27. 	Siegler IA, Bazile C, Warren WH. Mixed control for perception and action: timing and error correction in rhythmic ball-bouncing. Exp Brain Res 226: 603–615, 2013. doi: 10.1007/s00221-013-3475-7.

28. 	Sternad D, Duarte M, Katsumata H, Schaal S. Bouncing a ball: tuning into dynamic stability. Journal of experimental psychology Human perception and performance 27: 1163–84, 2001. doi: 10.1037/0096-1523.27.5.1163.

29. 	Avrin G, Siegler IA, Makarov M, Rodriguez-Ayerbe P. Model of rhythmic ball bouncing using a visually controlled neural oscillator. Journal of Neurophysiology 118: 2470–2482, 2017. doi: 10.1152/jn.00054.2017.

30. 	Fortineau V, Makarov M, Rodriguez-Ayerbe P, Siegler IA. Interactive robotics for human impedance estimation in a rhythmic task. 2020 IEEE 16th International Conference on Automation Science and Engineering (CASE). 2020, p. 1043–1048.

31. 	Fortineau V, Siegler IA, Makarov M, Rodriguez Ayerbe. Human arm impedance estimation in dynamic human-robot interaction, a preliminary study. Computer Methods in Biomechanics and Biomedical Engineering 24: S256, 2021. doi: 10.1080/10255842.2021.1978758.

32. 	Fortineau V, Makarov M, Rodriguez-Ayerbe P, Siegler IA. Towards a seamless experimental protocol for human arm impedance estimation in an interactive dynamic task. 2021 30th IEEE International Conference on Robot Human Interactive Communication (RO-MAN). 2021, p. 31–36.

33. 	Lamy X, Colledani F, Geffard F, Measson Y, Morel G. Achieving efficient and stable comanipulation through adaptation to changes in human arm impedance. 2009 IEEE International Conference on Robotics and Automation. 2009, p. 265–271.

34. 	Kawazoe Y. Dynamics of the Ball-Racket Impact in Tennis : Contact Force, Contact Time, Coefficient of Restitution, and Deformation. The Proceedings of the Asian Conference on Multibody Dynamics 2002: 286–293, 2002. doi: 10.1299/jsmeacmd.2002.286.

35. 	Tsuji T. Human Arm Impedance in Multi-Joint Movements. In: Advances in Psychology, edited by Morasso P, Sanguineti V. North-Holland, p. 357–381.

36. 	Erden MS, Billard A. End-Point Impedance Measurements Across Dominant and Nondominant Hands and Robotic Assistance with Directional Damping. IEEE Transactions on Cybernetics 45: 1146–1157, 2015. doi: 10.1109/TCYB.2014.2346021.

37. 	Kurtzer IL, Pruszynski JA, Scott SH. Long-Latency Reflexes of the Human Arm Reflect an Internal Model of Limb Dynamics. Current Biology 18: 449–453, 2008. doi: 10.1016/j.cub.2008.02.053.

38. 	Libet B. Unconscious cerebral initiative and the role of conscious will in voluntary action. Behavioral and Brain Sciences 8: 529–539, 1985. doi: 10.1017/S0140525X00044903.

39. 	Wei K, Dijkstra TMH, Sternad D. Stability and Variability: Indicators for Passive Stability and Active Control in a Rhythmic Task. Journal of Neurophysiology 99: 3027–3041, 2008. doi: 10.1152/jn.01367.2007.

40. 	Huynh H, Feldt LS. Estimation of the Box Correction for Degrees of Freedom from Sample Data in Randomized Block and Split-Plot Designs. Journal of Educational Statistics 1: 69–82, 1976. doi: 10.3102/10769986001001069.

41. 	Mauchly JW. Significance Test for Sphericity of a Normal n-Variate Distribution. The Annals of Mathematical Statistics 11: 204–209, 1940. doi: 10.1214/aoms/1177731915.

42. 	Morice AHP, Siegler IA, Bardy BG, Warren WH. Learning new perception–action solutions in virtual ball bouncing. Exp Brain Res 181: 249–265, 2007. doi: 10.1007/s00221-007-0924-1.

43. 	Hogan N, Sternad D. On rhythmic and discrete movements: reflections, definitions and implications for motor control. Exp Brain Res 181: 13–30, 2007. doi: 10.1007/s00221-007-0899-y.

44. 	Schaal S, Sternad D, Osu R, Kawato M. Rhythmic arm movement is not discrete. Nat Neurosci 7: 1136–1143, 2004. doi: 10.1038/nn1322.

45. 	Degallier S, Ijspeert A. Modeling discrete and rhythmic movements through motor primitives: a review. Biol Cybern 103: 319–338, 2010. doi: 10.1007/s00422-010-0403-9.

46. 	Sternad D, Dean WJ, Schaal S. Interaction of rhythmic and discrete pattern generators in single-joint movements. Human Movement Science 19: 627–664, 2000. doi: 10.1016/S0167-9457(00)00028-2.

47. 	Kuxhaus L, Zeng S, Robinson CJ. Dependence of elbow joint stiffness measurements on speed, angle, and muscle contraction level. J Biomech 47: 1234–1237, 2014. doi: 10.1016/j.jbiomech.2013.12.008.

48. 	Piovesan D, Pierobon A, DiZio P, Lackner JR. Measuring multi-joint stiffness during single movements: numerical validation of a novel time-frequency approach. PLoS One 7: e33086–e33086, 2012. doi: 10.1371/journal.pone.0033086.